Course: CSE 462 - Network Analysis and Design LAB 2 – Switch Networks

| Group ID | Group 1 |
| --- | --- |
| Student Name | Ngô Quang Khiêm, Hà Quang Minh, Lê Quốc Khánh |
| Final Score |  |

Lab Exercise Submission

Students are responsible for submitting the final report by the stated deadline for full marks. Late submissions will NOT be accepted.

Objective: This lab is to guide students how to build switch networks.

NOTE: Students should read the guideline carefully before conducting Lab experiments.

🙡 - Good luck - 🙣

# TASK 1 – HOW TO RESET SWITCH

Switch>enable

Switch#delete flash:vlan.dat

Delete filename [vlan.dat]?[Enter]

Delete flash:vlan.dat? [confirm] [Enter]

If there was no VLAN file, this message is displayed.

%Error deleting flash:vlan.dat (No such file or directory)

Switch#erase startup-config

Erasing the nvram filesystem will remove all files! Continue? [confirm] Enter

Erase of nvram: complete

Use the reload command to reboot the switch:

Switch#reload

System configuration has been modified. Save? [yes/no]: n

Proceed with reload? [confirm] [Enter]

Reload requested by console.

Would you like to enter the initial configuration dialog? [yes/no]: n

Press RETURN to get started! [Enter]

Switch1

Switch2

Result

# TASK 2 – BASIC SWITCH NETWORK

Figure 2.1. Basic switch network diagram

Make device connections according to the above model.

Note: Delete old configuration information existing in the devices before proceeding

Step 1: Make connections according to the diagram and configure basic parameters for the Switch (See lab 1)

- Set hostname, banner

- Set enable pass as “eiu@123”

-  Configure and set password for telnet sessions, password is “cisco”

Step 2: Configure IP for switches and PCs

PC1: 192.168.10.11

PC2: 192.168.10.12

PC3: 192.168.10.23

To configure the IP for the Switch, you must go to interface vlan1, and the IP on the switch only is for the management purpose.

EIU01(config)#interface vlan 1

EIU01(config-if)#ip address 192.168.1.10 255.255.255.0

EIU01(config-if)#no shutdown

EIU01(config-if)#exit

EIU01(config)#ip default-gateway 192.168.1.1

EIU01#copy running-config startup-config

NOTE: Do the same on the remaining devices

Step 3: Check configuration information:

EIU01#show running-config

EIU01#show vlan

EIU01#show mac-address-table

SWITCH 1:

SWITCH 2:

Execute the command to delete old information in the MAC address table:

EIU01#clear mac-address-table

Then review the information in the MAC address table:

EIU01#show mac-address-table

Step 4: Check the connection between PCs and between PC and Switch

- Use the Ping command from PC1 to the switch, and other PCs in the model.

- Observe the Ping process. If it fails, you must check configuration again.

- From PC1, telnet to the Switches in the model.

PC3-PC1&PC2

PC3-SWITCH 1

PC3-SWITCH 2

TELNET

# TASK 3 – EXTENTED SWITCH NETWORK

Figure 2.2. Expanded switch network diagram.

Step 1: Make connections according to the diagram and configure basic parameters for the Router (See lab 1)

- Set hostname, banner

- Set enable pass as “eiu@123”

-  Configure and set password for telnet sessions, password is “cisco”

Step 2: Configure IP for switches, routers and PCs

In the above model, can it work without a Router device? What is the role of the Router in the above model?

Answer:

PC3:

Step 3: Check configuration information:

#show running-config

#show vlan

#show ip route

#show ip interface brief

Step 4: Check the connection between PCs, Switch and Router

- Use the Ping command from PC1 to the switch, and other PCs in the model.

- Observe the Ping process. If it fails, you must check again.

- From PC, telnet to the Switches in the model.

PING PC2 -> ROUTER AND SWITCH2

TELNET

# TASK 4 – VLAN and TRUNKING

Figure 2.3. Switch network diagram with VLAN and trunking

    - VLAN 10: port 4, 5, 6

    - VLAN 20: port 7, 8, 9

    - Trunk: port 15

Step 1: Make connections according to the diagram and configure basic parameters for the Switch (See lab 1)

    - Set hostname, banner

    - Set enable pass as “eiu@123”

    - Configure and set password for telnet sessions, password is “cisco”

Step 2: Assign IP addresses to PCs  Example PC11:

- IP address: 192.168.10.11

- Subnet mask: 255.255.255.0

Do the appropriate configuration for other PCs

- Vlan 10: PC1

- Vlan 20: PC2

pc3 (vlan 10):

Step 3: Create VLANs on SWITCHs

Switch EIU01

EIU01#show vlan

EIU01(config)#vlan 10

EIU01(config-vlan)#name Staff

EIU01(config)#vlan 20

EIU01(config-vlan)# name IT

EIU01#show vlan

Do the same for switch EIU02

Step 4: Assign ports to VLANs

Switch EIU01:

Assign ports Fa0/4, Fa0/5, Fa0/6 to VLAN 10

EIU01(config)#interface fastethernet 0/4

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

EIU01(config-if)#interface fastethernet 0/5

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

EIU01(config-if)#interface fastethernet 0/6

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

Do the same for VLAN 20 and on Switch EIU02

EIU01#show vlan

EIU01#show running-config

Check the configuration information again:

Step 5: Check the connection between PCs

- Can PC11 ping PC12? Why? Answer:

Yes, because it’s in the same switch.

2.    Can PC11 ping PC13? Why? Answer:

No, we have to turn on the trunk gate.

Step 6: Create Trunk port with Encapsulation dot1q

Switch EIU01:

EIU01(config)#interface fastethernet 0/15

EIU01(config-if)#switchport mode trunk

EIU01(config-if)#switchport trunk encapsulation dot1q

EIU01(config-if)# switchport trunk allowed vlan 10,12,13

Do the same with switch EIU02

Step 7: Check information on the Trunk interface with the following commands:

#show interface fastethernet 0/15  switchport

#show interface fastethernet 0/15

#show running-config

#show vlan

Step 8: Check the connection between PCs

1.  Can PC11 ping PC13? Why?

2. Can PC11 ping PC24? Why?

3. If the PING was failed, suggest a solution to PING successfully.

Answer:

https://mdungblog.wordpress.com/cau-hinh-telnet-ssh-cho-switch-cisco/

Task 4 : On Cisco Packet Tracer

VLAN AND TRUNKING

    - VLAN 10: port 4, 5, 6

    - VLAN 20: port 7, 8, 9

    - Trunk: port 15

Step 1: Make connections according to the diagram and configure basic parameters for the Switch (See lab 1)

    - Set hostname, banner

    - Set enable pass as “eiu@123”

    - Configure and set password for telnet sessions, password is “cisco”

Answer :

Switch EIU01

Switch EIU02

Step 2: Assign IP addresses to PCs  Example PC11:

- IP address: 192.168.10.11

- Subnet mask: 255.255.255.0

Do the appropriate configuration for other PCs

PC11 :

PC12

PC13

PC14

PC21

PC22

PC23

PC24

Step 3 : Create Vlans on Switchs

Step 4: Assign ports to VLANs

Switch EIU01:

Assign ports Fa0/4, Fa0/5, Fa0/6 to VLAN 10

EIU01(config)#interface fastethernet 0/4

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

EIU01(config-if)#interface fastethernet 0/5

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

EIU01(config-if)#interface fastethernet 0/6

EIU01(config-if)#switchport mode access

EIU01(config-if)#switchport access vlan 10

Do the same for VLAN 20 and on Switch EIU02

EIU01#show vlan

EIU01#show running-config

EIU01

EIU02

Làm cho Vlan 20

EIU01

EIU02

- Can PC11 ping PC12? Why? Answer:

Yes, because it’s in the same switch.

2.    Can PC11 ping PC13? Why? Answer:

No, we have to turn on the trunk gate.

Step 6 : Create trunk port with Encapsulation dot1q

Step 7: Check information on the Trunk interface with the following commands:

#show interface fastethernet 0/15  switchport

#show interface fastethernet 0/15

#show running-config

#show vlan

Step 8: Check the connection between PCs

1.  Can PC11 ping PC13? Why?

Yes, because it’s in the same Vlan 10 and we have turn on trunk gate.

2. Can PC11 ping PC24? Why?

No, because it’s in a different Vlan: PC11 in Vlan 10 and PC24 in Vlan 20.

3. If the PING was failed, suggest a solution to PING successfully.

Option 1: Use a router to connect the router to the switches and configure sub-interfaces for each VLAN.

Option 2: Use a Layer 3 switch to configure SVI for VLAN 10 and VLAN 20 on the Layer 3 switch to route between VLANs.

Remember