Course: CSE 462 - Network Analysis and Design LAB 4 – Routing Configuration

| Group ID | Group 1 |
| --- | --- |
| Student Name | Ngô Quang Khiêm, Hà Quang Minh, Lê Quốc Khánh |
| Final Score |  |

Lab Exercise Submission

Students are responsible for submitting the required working files by the stated deadline for full marks. Late submissions are NOT accepted.

Objective: This lab is to guide you how to work with Routers and routing protocols.

NOTE: Students should read the guideline carefully before conducting Lab experiments.

🙡 - Good luck - 🙣

TASK 1 – CONFIGURE STATIC ROUTE

Step 1: Configure basic parameters for Routers R1, R2, ISP

- Set hostname, banner

- Set enable pass as “eiu@123”

- Configure and set password for SSH sessions, password is “cisco@123”

R1:

R2:

ISP:

PC1:

PC2 : 192.168.3.30 /24

Step 2: Assign IP addresses to Routers and PCs

Note: On the ISP, create a loopback interface (virtual interface - used to test results).

ISP(config)#interface loopback 0

ISP(config-if)#ip address 203.162.0.1 255.255.255.0

There is no need to type the no shutdown command for this interface.

Step 3: Configure static routes for R1

From the network model above, it must be determined which networks are not directly connected to R1. From there, execute the following static route commands:

R1(config)#ip route 192.168.3.0 255.255.255.0 192.168.1.2

Do the same for other networks

Step 4: Configure static routes for R2

Must determine which networks are not directly connected to R2, then execute the following static route commands:

R2(config)#ip route 192.168.2.0 255.255.255.0 192.168.1.1

According to the figure, because R2 acts as a Gateway for the Networks on the left (R1) to go to the ISP service provider, we can use the following default route command:

R2(config)#ip route 0.0.0.0 0.0.0.0 serial 0

Step 5: Configure static routes for ISP Routers

From the figure, you must determine which networks are not directly connected to the ISP Router. From there, execute the following static route commands:

ISP(config)#ip route 192.168.2.0 255.255.255.0 172.16.0.1

ISP(config)#ip route 192.168.3.0 255.255.255.0 172.16.0.1

ISP(config)#ip route 192.168.1.0 255.255.255.0 172.16.0.1

Step 6: Check the routing tables on the routers, and other parameters:

Show ip protocol

Show ip int brief

Show ip route

ISP:

R1:

R2:

Check connection between devices in networks with the following commands:

1.  Use the Ping command to check the connection between PCs.

PING PC2 →  PC1 :

PING PC1 → PC2:

2. Use the Traceroute command to check the connection between Routers.

3. Use the Telnet command to check the application layer between PC and Router.

TASK 2 – BROADCAST ROUTING INFORMATION BETWEEN RIP AND STATIC ROUTE

Note: Make device connections according to the above model. You can delete the old configuration before doing this Lab.

Step 1: Make connections according to the diagram and configure basic parameters for the Switch.

- Set hostname, banner

- Set enable pass as “eiu@123”

- Configure and set password for SSH sessions, password is “cisco@123”

Step 2: Configure IP for switches and PCs

Step 3: Configure RIP dynamic routing protocol for R1, R2

R1(config)#router rip

R1(config-router)#network 192.168.1.0

R1(config-router)#network 192.168.2.0

Similar configuration for router R2

R1:

R2:

Step 4: Configure static routes for ISP Routers

The ISP router acts as a service provider for the internal network system R1 and R2. Due to security requirements, the ISP router does not implement any dynamic routing protocols with the internal network. Therefore, use static route with internal network R1, R2.

ISP(config)#ip route 192.168.1.0 255.255.255.0 172.16.0.1
ISP(config)#ip route 192.168.2.0 255.255.255.0 172.16.0.1
ISP(config)#ip route 192.168.3.0 255.255.255.0 172.16.0.1

Step 5: Check the connection between networks:

From PC 192.168.2.20 on R1's LAN, perform a ping command to the following addresses:

PC1_C:\ping 192.168.1.2	🡪 must be successful!!

PC1_C:\ping 192.168.3.1	🡪 must be successful!!

PC1_C:\ping 172.16.1.1

PC1_C:\ping 203.162.0.1 ???

On 2 Routers R1 & R2, execute the show ip route command to see if any neighboring networks have been learned through the RIP protocol. If not, check the configuration, cable connection, etc.

#show ip route

Networks learn through the RIP protocol?

R1:

R2:

Similarly, from PC 192.168.3.30 on R2's LAN, ping the interface addresses. However, everyone cannot ping the ISP, especially the loopback interface address 203.16.0.1.

Step 6: Configure Default information broadcast to R2

Router R2 in the diagram acts as the Gateway for R1 and the Networks on the left of R2. Therefore, in order for these networks to go outside the internet, R2 must take on the function of broadcasting Default information to internal networks.

R2(config-router)#default-information originate

R2(config-router)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 172.16.0.2

After configuring this command, execute the show ip route command on the Routers to see how the routing table has changed?

R1:

R2:

Ping from PC to ISP loopback interface to check:

PC1_C:\ping 203.162.0.1  🡪 must be successful!!

Step 7: Save configuration to NVRAM

ISP#copy running-config startup-config

R1#copy running-config startup-config

R2#copy running-config startup-config

Commands used to check configuration information:

#Show ip protocol

#Show ip int brief

#Show ip route

#clear ip route *

R1:

R2:

ISP:

TASK 3 – BROADCAST ROUTING INFORMATION BETWEEN OSPF AND STATIC ROUTE:

Step 1 & 2: Configure basic parameters for 3 Routers and assign IP addresses to interfaces

R1:

R2:

ISP:

Step 3: Configure OSPF on R1 and R2

R1(config)#router ospf 1

R1(config-router)#network 192.168.1.0 0.0.0.255 area 1

R1(config-router)#network 192.168.2.0 0.0.0.255 area 1

Do the same for R2

PC1:

PC2:

Step 4: Configure static routes for ISP Routers

The ISP router acts as a service provider for the left network model (R1-R2). Therefore, security on the ISP is necessary, so the ISP router does not use dynamic routing protocols but uses static routing.

Configure static routing on the ISP router to the networks of routers R1 and R2.

ISP(config)#ip route xxx  . . .

Step 5: Check connection between networks:

On 2 Routers R1 & R2, execute the show ip route command to see if any neighboring networks have been learned through the OSPF protocol. If not, then check the configuration, cable connection, etc.

Which Networks Learn Through the OSPF Protocol?

R1:

R2:

From PC1 192.168.2.20 on R1's LAN, perform a ping command to the following addresses:

PC1_C:\ping 192.168.1.2	🡪 must be successful!!

PC1_C:\ping 192.168.3.1	🡪 must be successful!!

PC1_C:\ping 172.16.0.2

PC1_C:\ping 203.162.0.1 ???

Similarly, from PC 192.168.3.30 on R2's LAN, ping the interface addresses. However, everyone cannot ping the ISP, especially the loopback interface address 203.162.0.1.

Step 6: Configure Default information broadcast to R2

R2 in the diagram acts as the Gateway for R1 and the Networks on the left of R2. Therefore, in order for these networks to go outside the internet, R2 must take on the function of broadcasting Default information to internal networks.

R2(config-router)#default-information originate

R2(config-router)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 172.16.0.2

After configuring this command, execute the show ip route command on the Routers to see how the routing table has changed?

R1:

R2:

Meaning of default-information originate command?

This command is typically done to inform other routers about how to reach external networks or the internet.

Step 7: Answer the question

- How long does the OSPF protocol periodically send update information?

OSPF does not send periodic updates like RIP. Instead, OSPF uses LSAs and only sends updates when there is a topology change in the network. However, OSPF does periodically refresh its LSAs every 30 minutes (1800 seconds) by default.

- What is the administrative distance value of OSPF?

The administrative distance of OSPF is 110. This value determines the trustworthiness of OSPF routes compared to routes learned from other protocols

- How many networks are learned by the OSPF protocol on Router R1?

Step 8: Configure password authentication mechanism in OSPF

On Router 1:

R1(config)#interface serial 0

R1(config-ip)#ip ospf authentication-key eiu@123

R1(config-ip)#exit

R1(config)#router ospf 1
R1(config-router)# area 1 authentication

- Comment on what changes in the routing table between the 2 Routers?

R1:

R2:

- Perform ping command between hosts, is it successful?

- Why?

When configuring OSPF authentication, routers in the same area 1 must use the same authentication-key to exchange OSPF information.

If another router in area 1 has not been configured for authentication or has a different key (eiu@123), OSPF cannot establish a neighbor relationship, resulting in a failure to ping because the routing table is not updated.

- How many routes (subnets) are learned by the OSPF protocol on Router R1?

On Router 2:

R2(config)#interface serial 1

R2(config-ip)#ip ospf authentication-key eiu@123

R2(config-ip)#exit

R2(config)#router ospf 1
R2(config-router)# area 1 authentication

- Comment on what changes in the routing table between the 2 Routers?

R1:

R2:

- Perform ping command between hosts, is the result successful?

- If ping is not successful, recheck the configuration above.